# Geohazard Area DPA Guidelines

# Checklist to be completed and signed by Qualified Geotechnical Engineer

Report Guidelines

| Report Requirements | Met? |
| --- | --- |
| Describe the topography and geomorphology of the site |  |
| Identify potential natural hazards |  |
| Assess the nature, extent, frequency, and potential effects of identified hazards |  |
| Include a review of relevant previous geohazard studies or engineering work in the vicinity |  |
| Identify safe building sites |  |
| Recommend hazard mitigation measures and development protection measures |  |
| Recommend the conditions and requirements of the development permit |  |
| Consider Provincial requirements for flood and landslide assessment |  |
| Consider the 2010 APEGBC Guidelines for Legislated Landslide Assessments for Proposed Residential Developments, or any subsequent updated version |  |
| Certify that the land may be used safely for the intended use |  |
| Information must be detailed enough to support registration of a Section 219 Covenant as required by the Land Title Act. |  |

## Slope Hazard Guidelines

| Guidelines – Toe of Slope | Met? |
| --- | --- |
| Qualified professionals must verify the adequate minimum setback from the toe of slopes, based on slope stability and/or runout modelling. |  |
| Professional engineers and/or professional geoscientists must verify whether the minimum 15 m setback from the crest of slopes is adequate. Site-specific analysis may identify areas, such as the northeast portion of the Town, where a greater setback is required. |  |
| Guidelines – Crest of Slope | Met? |
| Where a reduced setback of less than 15 m (<15 m) from the crest of the slope is proposed, the qualified professional must provide a defensible justification based on: - Subsurface investigations. - Groundwater monitoring (including piezometers). - Slope stability modelling - Site-specific geometry and soil/groundwater conditions |  |
| The following professional guidelines must be adhered to: - EGBC Professional Practice Guideline: Geotechnical Engineering Services for Building Projects, including application of the Canadian Foundation Engineering Manual (CFEM) or provision of rationale where deviation occurs. - EGBC Professional Practice Guideline: Landslide Assessments in British Columbia. |  |
| Standard practice is expected to include: - Site reconnaissance - Subsurface investigation and reconciliation with literature review findings - Groundwater monitoring - Slope stability and/or runout modelling. |  |

## Creek Flood Hazard Guidelines

| Guidelines | Met? |
| --- | --- |
| Qualified professionals must verify a safe development site within the creek flooding areas. Analysis to support development in a flood area may include: - Hydraulic modelling to identify flood extents and the capacity of the culverts in the vicinity. The modelling may need to account for debris flows. i. Gibson Creek – KWL (2013) identified that Gibson Creek may experience debris flows based on the Melton Ratio. - Slope stability and/or runout modelling. |  |

## Coastal Flood Hazard Guidelines

| Guidelines | Met? |
| --- | --- |
| Development within the Coastal Flood Hazard areas must include a coastal engineering report prepared by a qualified Professional Engineer with coastal flood experience (i.e. Coastal Engineer) to verify a safe development site. |  |
| A property-specific Flood Construction Level (FCL) is required if any proposed livable floor areas will be lower than the FCLs provided in Table 1, pg. 8 of Schedule C. |  |
| Analysis to support development in the coastal flooding areas must include: - Estimation of coastal flood levels Consideration of future sea level rise and wave run-up - Compliance with Provincial Flood Hazard Guidelines - Mitigation measures from potential transfer of flood risk to adjacent properties, consistent with the EGBC Flood Hazard Assessment (FHA) Guidelines (2018) Consideration of nature-based solutions or Greenshores approaches that manage coastal flood and erosion |  |
| Development in Coastal Flood Hazard Areas must identify a Flood Construction Level (FCL) sufficient for the intended lifespan of the development (or adhere to minimum FCLs in Table 1) and specify measures to achieve it, such as: - Engineered fill - Raised foundations - Coastal bank protection or armouring. |  |